Maintenance Inspection Checklist

| EXTERNAL |  |  |
| --- | --- | --- |
| Area of inspection | What to look for? | Comments |
| Roof areas generally | Is there any loss or damage to the roof coverings? |  |
| Slate roofs | Is there debris from broken slates / tiles on the ground |  |
|  | Are there any loose, slipped, or missing slates? |  |
| Lead flashing | Are there any splits in flashings or holes in associated mortar pointing? |  |
| Rainwater goods | Are any of the gutters blocked? |  |
|  | Is there staining below gutters & behind downpipes suggesting blocked or damaged sections? |  |
|  | Are the gulleys blocked by leaves or debris? |  |
|  | Does water drain away slowly from gullies after rainfall, suggesting blocked or damaged drain runs? |  |
|  | Are paint finishes peeling or flaking or signs of corrosion? |  |
|  | Is there excessive moss or plant growth at the base of the wall or to the joints of ground gutters? |  |
|  | Are any large trees near building/touching the building or overhanging the roof? |  |
| External walls | Are there any cracks or other signs of movement? |  |
|  | Are there any areas of masonry that have become badly eroded? |  |
|  | Is there any plant growth on the walls that may cause damage? |  |
|  | Are any air bricks or vent grilles obstructed by vegetation or raised ground levels? |  |
| Windows and doors | Is there any cracking or de-lamination of the stonework to the surrounds or tracery |  |
|  | Are there leaded lights or stained glass panels that need attention? |  |
|  | Are there any signs of decay or insect infestation? |  |
|  | Are locks, bolts and hinges stiff/rusting? |  |
| INTERNAL |  |  |
| Internal walls | Are there any patches of staining or other signs of excessive dampness? |  |
|  | Are there any patches of damaged or flaking plaster or paint? |  |
| Fixtures and fittings | Are there any signs of timber decay or insect attack on the pews, panelling or other joinery including cupboards? |  |
| Windows and doors | Do you need to use force to open or close windows/doors? |  |
|  | Are there any broken, cracked or missing panes of glass? |  |
| Ceiling | Are there any cracks or other signs of movement? |  |
|  | Are there patches of damp or staining on the roof or ceiling? |  |
| Floors | Do any floor coverings show signs of excess wear and tear or movement that may present a hazard? |  |
|  | Are any air bricks/vents at the base of the walls blocked, restricting air flow? |  |
|  | Are there any signs of timber decay or insect attack to floors or pews? |  |
| Building services | Do the lightening conductors need checking? |  |
|  | Do electrical appliances need testing and marking as such? |  |
|  | Does the fire risk assessment need updating and do your fire extinguishers need checking? |  |
|  | Does the heating system need to be checked to ensure it is operating correctly? |  |
|  | Does your first aid kit need checking and updating? |  |